Data-Driven Estimation and Forecasting of Chlorophyll-a in a Patagonian Lake Using Satellite Observations and Machine Learning

Lien Rodríguez-López1, David Francisco Bustos Usta2, Lisandra Bravo Alvarez3 , Luc Bourrel4, Frederic Frappart5 and Roberto Urrutia6

1 Facultad de Ingeniería, Universidad San Sebastián, Lientur 1457, Concepción 4030000, Chile; lien.rodriguez@uss.cl

2Facultad de Oceanografía, Universidad de Concepción, Concepción 4030000, Chile; davidbustos@udec.cl

3Department of Electrical Engineering, Universidad de Concepción, Edmundo Larenas 219, Concepción 4030000, Chile; lisanbravo@udec.cl

4 Géosciences Environnement Toulouse, UMR 5563, Université de Toulouse, CNRS-IRD-OMP-CNES, 31000 Toulouse, France; luc.bourrel@ird.fr

5 INRAE, Bordeaux Sciences Agro, UMR 1391 ISPA, 33140 Villenave-d’Ornon, France; frederic.frappart@inrae.fr

6Facultad de Ciencias Ambientales, Universidad de Concepción; rurrutia@udec.cl

* Correspondence: lien.rodriguez@uss.cl

Highlights:

- TimesFM and Chronos Large produced the strongest short-horizon forecasts from univariate Chl-a context windows, with D1 MAE between 0.063 and 0.106 µg/L across the two stations.

- Forecast skill degraded with horizon: D7 forecasts retained the broad seasonal envelope, while D14–D28 forecasts showed increasing amplitude damping and timing uncertainty.

Abstract:

This study evaluates short-term forecasting of chlorophyll-a (Chl-a) dynamics at two monitoring stations in Lake Villarrica, northern Patagonia, Chile, using reproducible station-level Chl-a time series and foundation time-series models. The forecasting experiments used univariate Chl-a context windows as inputs to TimesFM and Chronos Large and evaluated 1–30 day horizons under a rolling-origin design. Short-horizon skill was highest at the 1-day horizon: at La Poza, D1 MAE was 0.099 µg/L for TimesFM and 0.106 µg/L for Chronos Large; at Pucón, D1 MAE was 0.063 µg/L for TimesFM and 0.071 µg/L for Chronos Large. Forecast error increased with lead time, with D28 MAE ranging from 0.606 to 0.757 µg/L across stations and models. Lag diagnostics showed that most D7 forecast cases had maximum correlation at lag 0, although Pucón Chronos Large showed a non-zero best lag, indicating that peak timing remains uncertain for warning applications. We also evaluated q10/q50/q90 predictive intervals and observed/gap-stratified performance. The results support the use of foundation time-series models for short-term forecasting of reconstructed Chl-a products, while highlighting limitations due to imputation, missing exogenous drivers, and the absence of validated satellite-feature ingestion in the current forecast architecture.

Keywords: algal pigments; Remote Sensing; machine learning; Nord Patagonian Lakes

- Introduction

Algae blooms, also known as blooms, are events of accelerated growth of algal communities in aquatic systems, which can significantly alter water quality and ecological balances (Feng et al., 2024). These blooms, often dominated by species of cyanobacteria or microalgae, are closely related to the increase in nutrients (mainly nitrogen and phosphorus), the increase in water temperature, and the reduction of the flow or renewal of the lake or fluvial body (Bilous et al., 2023). Among the main biogeochemical indicators of these blooms are photosynthetic pigments, such as Chlorophyll-a (present in all algae), phycocyanin (characteristic of cyanobacteria) and other accessory pigments such as carotenoids and fucoxanthin, whose presence allows differentiation between algal groups (Chini Zittelli et al., 2023; Wang and Convertino, 2023).

Traditional monitoring of these pigments has relied on in situ sampling and laboratory analysis, methods that, while accurate, are costly, punctual, and have low spatial coverage (Ma et al., 2025; Singh et al., 2024). In this context, remote sensing has emerged as a complementary and powerful alternative for environmental monitoring (Mahmoud et al., 2025; Rolim et al., 2023; Wu et al., 2025). Using optical sensors mounted on satellite, aerial, or terrestrial platforms, it is possible to obtain spectral information of the water surface, which is related to the inherent optical properties of water (IOPs) and the concentration of pigments present in the water column (Jaywant and Arif, 2024; Manzi, 2025; Srivastava et al., 2023). The analysis of these spectral signals allows the identification and estimation of the abundance of algal biomass in large areas, with a high periodicity and at a low cost per unit area (Arias et al., 2025; Baek and Kim, 2025; Raghul and Porchelvan, 2025).

However, the interpretation of spectral signals in aquatic environments presents significant challenges due to the high variability of the optical components of water, such as dissolved organic matter, suspended sediments and different types of phytoplankton (Dierssen et al., 2021; Wu et al., 2025). To address this complexity, classical approaches based on spectral indices or simple statistical models are progressively being replaced or complemented by methodologies based on machine learning (He et al., 2025; Li et al., 2025). These techniques, which include algorithms such as decision trees, support vector machines (SVMs), neural networks and assembly models among the most used, allow the construction of nonlinear relationships between spectral and biophysical variables, improving the accuracy and generalizability of estimates of pigments and types of algae (Ananias and Negri, 2021; Beal et al., 2024; Sonmez et al., 2022).

In southern Chile, Lake Villarrica is one of the most emblematic and monitored lake ecosystems due to its ecological, economic and social importance (Aranda et al., 2021). Located in the Araucanía Region, this lake of glacial origin is a key tourist and recreational pole for the area, as well as being a vital source of water supply and a cultural symbol of the Mapuche territory (Molares et al., 2022). However, during the last decades, it has experienced a growing deterioration in water quality, with recurrent episodes of cyanobacteria blooms, especially in the summer season, which has generated concern among the scientific community, environmental authorities and the local population (Rodríguez López et al., 2025; Rodríguez-López et al., 2024, 2023a). However, the optical complexity of Lake Villarrica, because of the mixture of volcanic waters, urban discharges and fluvial contributions, poses challenges to traditional models based on spectral indices or empirical algorithms (Rodríguez López et al., 2025). To overcome these limitations, the use of machine learning techniques has gained prominence in recent research (Park et al., 2024; Xiong et al., 2022; Yang and Huang, 2023). These methodologies allow the construction of complex multivariate models that capture nonlinear relationships between spectral variables and concentrations of pigments such as Chlorophyll-a or phycocyanin, improving the accuracy of the estimates and the discriminating capacity of different types of algae (Che et al., 2023; Khruschev et al., 2022; Lima et al., 2023).

The objective of our work is the integration of remote sensing with machine learning tools in Lake Villarrica to estimate the concentrations of Chlorophyll-a and phycocyanin. We hope to propose a robust approach for automated surveillance of algal blooms, allowing not only to quantify concentrations of key pigments, but also to generate early warnings and model spatiotemporal trends. These capacities are especially relevant in the current context of climate change and increasing anthropogenic pressure on aquatic ecosystems, as they provide fundamental data for the sustainable management of water resources, the protection of biodiversity and the prevention of impacts on human health.

2. Materials and Methods

2.1 Study area: main characteristics

Lake Villarrica, located in the Araucanía Region of southern Chile, is one of the most emblematic lake systems in the country, both for its ecological relevance and for its socioeconomic importance (Rodríguez-López et al., 2022). It is located at the foot of the Villarrica volcano, which gives it a landscape of high tourist and cultural value. With an approximate surface area of 176 km² and a maximum depth of close to 160 meters, this lake of glacial origin is part of the hydrographic basin of the Tolten River. Its hydrological regime is influenced by seasonal rainfall, Andean melting and underground inputs, which makes it a dynamic ecosystem sensitive to climatic and anthropogenic variations.

In ecological terms, the lake is of marked scientific interest due to the recurrent presence of harmful algal blooms (HABs) during the last decades, a phenomenon associated with eutrophication and pressures derived from land use in the basin, the urbanization of its banks and intensive tourist activities (Rodríguez-López et al., 2023b). These processes have generated concern in local communities, public institutions and academia, since they affect water quality, aquatic biodiversity and the ecosystem services that the lake provides. Likewise, Villarrica is a natural laboratory for the monitoring and application of remote sensing and environmental modeling tools, since it integrates hydrological, biogeochemical and climatic variables that allow evaluation of global change and the management of water resources on mountain lake systems in the temperate zone of South America.

Figure 1. (a). Lake Villarrica in South America; (b) Digital Elevation Model of Araucanía Region and (c) Villarrica Lake. Red dots water sampling stations.

### 2.2. In-situ and Meteorological Data

Field campaigns were carried out to collect in-situ measurements of water quality parameters, including Chlorophyll-a (Chl-a), phycocyanin, Secchi depth, and nutrient concentrations between 1989 and 2024. This continuous, long-term monitoring network provided a robust baseline database consisting of 385 discrete limnological sampling events. Subsurface water samples (0.5 m depth) were collected using standard hydrographic Van Dorn bottles across established stations. For pigment quantification, water samples were filtered immediately through Glass Fiber Filters (GF/F, 0.7 µm pore size) under low vacuum, stored in the dark at -20°C, and analyzed via spectrophotometric extraction in 90% acetone in accordance with standard protocols validated by the Chilean Water Directorate (DGA). These campaigns were paired with vertical high-resolution profiles of temperature, conductivity, and dissolved oxygen acquired via calibrated multiparameter probes to monitor water column stratification

Sampling was conducted at multiple points across the lake to capture spatial variability and repeated seasonally to account for temporal fluctuations (see Figure 1). In parallel, meteorological data (air temperature, precipitation, wind speed, solar radiation, and relative humidity) were obtained from nearby weather stations and public climate databases (DMC, Chile). This information was used to evaluate external drivers of algal dynamics and to provide explanatory variables for statistical and machine learning models.

### 2.3 Satellite Data

Multispectral satellite imagery was used to complement field observations and provide synoptic coverage of the lake surface. Data from Sentinel-2 MSI and Landsat 8/9 OLI were acquired, atmospherically corrected, and processed to generate reflectance products. Spectral indices sensitive to phytoplankton pigments, such as Chlorophyll-a and phycocyanin, were computed to establish empirical and semi-empirical relationships with in-situ observations. Time-series analysis was applied to evaluate spatial-temporal patterns of algal blooms, while cloud masking and quality assurance procedures ensured reliable observations (Rodríguez-López et al., 2023b).

2.4. Data preparation and input structure

The raw water quality parameters time series from the Pucón and La Poza stations were subjected to a preprocessing pipeline to ensure temporal consistency, mitigate sensor noise, and optimize input feature representation for the machine learning architectures. The data processing workflow is summarized in Figure 2.

Figure 2. Sequence diagram for the preprocessing steps before ingesting data into models.

#### 2.4.1. Temporal Alignment and Climatological Imputation

Daily Chl-a targets were constructed from irregular station observations using a provenance-preserving preprocessing pipeline. For each station, source observations were first aggregated to daily means while retaining source-file paths, source rows, source counts, and quality flags. The resulting daily modeling series contains 941 days for La Poza and 1331 days for Pucón. Direct observations account for 432 days at La Poza (45.9%) and 740 days at Pucón (55.6%). Reconstructed/imputed values account for 527 days at La Poza (56.0%) and 685 days at Pucón (51.5%).

For the 2024 forecast evaluation target, direct observations were preserved exactly. Short bracketed gaps were filled using shape-preserving PCHIP interpolation only when valid neighboring observations existed. Longer gaps were reconstructed using a historical day-of-year baseline combined with harmonic seasonal fitting and analog residual structure. The final target records the imputation method for each value and retains flags for direct observations, missing-before-imputation, interpolated values, imputed values, outlier handling, smoothing, and source observation count. The final accepted target used for the refreshed foundation-model forecasts was not smoothed.

The time series used was divided into an 80–20% split: 80% (1440 days, between 1 January 2020 and 10 December 2023) of the data is used to establish a baseline for few-shot learning, and the remaining 20% (360 days, between 11 December 2023 and 4 December 2024)

2.4.2. Outlier Detection and Signal Smoothing

Potentially anomalous Chl-a values were flagged for review rather than automatically interpreted as either artifacts or true blooms. The pipeline records IQR-based outlier flags, abrupt-transition flags, high-Chl-a review flags, and whether a value was replaced in the modeling target. The generated QA table identified 18 La Poza and 94 Pucón direct observations requiring review before they can be treated as confirmed blooms or confirmed artifacts.

2.4.3. Feature Engineering and Cyclical Encoding

To enable the models to capture seasonal dependencies explicitly, the scalar date index was decomposed into numerical covariates [38]. Specifically, to address the discontinuity problem in cyclical time features (e.g., the jump from Month 12 to Month 1), we transformed the month variable into continuous coordinates using sine and cosine functions:

Msin=sin2π∙m12, Mcos=cos⁡(2π∙m12) (1)

Where m represents the month index. Additional extracted features included the year, day of the year, week of the year, and quarter.

2.1.4. Input Structure for Foundation Models

For the Zero-shot Foundation Models (TimesFM and Chronos), specific input formatting was required to leverage their pre-trained weights.

- Normalization: A Z-score analysis was performed on the target variable. If extreme outliers (z>3) were detected, a log⁡(1+x) transformation was applied to stabilize variance and reduce the impact of heteroscedasticity common in exponential algal growth [39].

- Context Window: The input context length was standardized to 1024 time steps (C=1024) to provide sufficient historical receptive fields for the attention mechanisms, with a prediction horizon set to 30 days.

- Quantile Mapping: To assess uncertainty, the models were configured to output probabilistic forecasts mapping to the 10th (low), 50th (median), and 90th (high) percentiles.

### 2.5 Model Architecture

We evaluated two pretrained foundation time-series models, TimesFM and Chronos Large, as zero-shot forecasters for daily Chl-a. Both models were applied to univariate Chl-a context windows; satellite reflectance bands or spatial predictors were not directly ingested by the forecast architectures in the reproducible experiments reported here. Forecasts were generated for horizons 1–30 days using a context length of up to 1024 daily values. The models returned point forecasts and q10/q50/q90 quantile forecasts where available. Runtime metadata, package versions, random seed, platform, and input-data hashes were stored with each model run to ensure reproducibility. Here is a detailed description of each model.

2.5.1 Chronos Model

The Chronos model architecture leverages the existing language model structures, such as T5 (Raffel et al., 2020), to perform time series forecasting without any complex domain-specific adjustments.

Considering that we have a time series given by X(1:C+H)=[x1,…,x(C+H)] where C represent the first-time steps to create an historical context and H the forecast horizon. The Chronos model follows these four steps to produce the forecast from a given univariate series [41]:

- Time series normalization: to avoid bias given by

Zi= xi-ms                          (2)

Where xi, represent the individual observations, m the mean and s the standard deviation.

- Quantization method: to convert the scaled time series into discrete tokens. For this we select B bin centres denoted by c1<….. <CB and B-1 edges bi separating them into (ci < bi < ci+1 for i ∈ {1,…, B-1}) . The quantization function q: R⟶{1,2,…, B} is defined by this equation:

qx= q,     if-∞≤x≤b12,     if b1≤x≤b21B,   bB-1≤x≤ ∞        (3)

Additional to the time series tokens {1,2,…,B} two special tokesn are also added (Vts) which are the PAD and EOS. The PAD is used to pad the time series of different lengths to a fixed length, and the EOS is used to denote the end of the sequence.

- Prediction: the model provides multiple realizations of the future using the predicted distribution using a loss function given by:

lθ= -h=1H+1i=1vts1ZC+h+1=1logpθZC+h+1=iz1:C+h       (4)

Where pθ(ZC+h+1=i|z1:C+h) represent the categorical distribution by the model parametrized by θ.

- Dequantization method: to obtain the final prediction, which implement this transformation d: {1,2,…, B}⟶ R defined in the following equation:

dj= cjx2i                                          (5)

2.5.2 Times FM Model

The Times FM (Time Series Foundation Model) is a pretrained (Das et al., 2024), decoder-only foundation model for time series forecasting. It is designed to generalize across domains, granularities, and forecast horizons without dataset-specific retraining.

Let the input time series be represented by:

X(1:C)=x1,…,xC,   xt ∈RD   (14)

where C is the number of historical time-steps and Dthe dimension of variables (often D=1 for univariate). The model predicts the next H time steps XC+1:C+H.

Times FM follows four key stages:

- Input normalization & Patching: Each time-series sequence is normalized (e.g., by mean and standard deviation) to remove scale bias. Then the normalized sequence is divided into contiguous non-overlapping patches of length pin. Each patch is processed through an MLP residual block, producing a token embedding

Formally:

Z=NormX1:C,   Z∈RCXD                         (15)

Partition into K=Cpin patches:

Pi= Zi-1pin+1:ipin,    i=1,…., k                 (16)

Then embed:

ei=MLPPi+posEnci,    ei ϵ Rdmodel (17)

- Decoder only transformer layers (Causal attention): The sequence of patch-embeddings e1,…,eK is fed through a stack of transformer layers configured for causal self-attention (i.e., each position attends only to previous ones) to respect temporal ordering

E(0)=[e1;e2;…;eK]                                         (18)

Then for each layer l=1,…, L

E(l)=TransformerLayer(l) (E(l-1)           (19)

- Output patch prediction & autoregressive decoding: Rather than predicting one time-step at a time, TimesFM uses an output patch length pout (which may be larger than the input patch length) to forecast multiple future steps in each decoding step

After obtaining the final layer representation E(L), a projection layer maps it to future patch predictions:

P (j)=Wo Ej(L)+bo,j=1,…,J,                 (20)

where each P (j)∈Rpout×D predicts the next pout points. These patches may be concatenated (and/or autoregressively fed back) to cover the horizon H.

- Loss function & Train objective: During pretraining, the model is optimized on a large corpus of time-series (≈100 billion points) using a point-forecast or optionally quantile loss (in later versions) over the predicted patches.

Formally, if the true future patches are {Ptrue (j)}, the loss might be:

L(θ)=1J pout Dj=1J∥Ptrue (j)-P (j)∥22                (21)

For probabilistic variants (in e.g., TimesFM 2.5) a quantile or CRPS-style loss may be used

2.6 Model evaluation

Forecasts were evaluated with a rolling-origin design over the 2024 target window. To keep foundation-model inference reproducible and computationally tractable, forecast origins were spaced every 14 days, and each origin generated horizons 1–30 days. Metrics were computed by station, model, and horizon using MAPE, MSE, RMSE, bias, median absolute error, and MAE. In addition to standard horizon metrics, we computed observed-only metrics, gap-stratified metrics, D7 lag diagnostics, threshold-warning confusion matrices, q10/q90 interval coverage, and a preliminary cross-site transfer baseline. Observed-only metrics were treated as sensitivity checks because the number of direct observed targets in the 2024 rolling-origin grid is small.

MAPE= 100%ni=1nyi-yiyi    (37)

MSE=1n i=1n(yi-yi)2     (38)

RMSE= 1n i=1n(yi-yi)2    (39)

BIAS= yi-yi    (40)

MedABS=medianyi-yi  (41)

MAE= 1n i=1n|yi-yi|     (42)

### 2.7 Algal Bloom Behavior Analysis

Finally, the validated models were used to analyze the spatio-temporal dynamics of algal blooms in the study lake. Patterns of bloom development, intensity, and decline were examined in relation to environmental drivers such as nutrient availability, meteorological forcing, and water column stratification. The integration of in-situ, meteorological, and satellite data with advanced modeling allowed for the identification of thresholds and critical conditions associated with bloom occurrence. This analysis provides insights into the ecological processes driving bloom behavior and supports the development of monitoring and early-warning strategies for lake management.

- Results

## In-situ Data Analyses of water quality

The figure 3 presents seasonal distributions of six key limnological variables turbidity (NTU), temperature (°C), dissolved oxygen (ppm), Chlorophyll-a (Chl-a, µgL⁻¹), dissolved organic matter (DOM, OD₄₅₀), and phycocyanin (µg L⁻¹) at two monitoring stations in Lake Villarrica (Pucón and La Poza). Each violin plot illustrates the range and variability of observations for summer, autumn, winter, and spring.

At Pucón, turbidity ranged from 1 to > 40 NTU, with the highest dispersion in summer, indicating greater sediment resuspension or algal biomass nearshore. Water temperature varied seasonally between 6 °C (winter) and 26–28 °C (summer), showing clear thermal stratification. Dissolved oxygen concentrations spanned 8–16 ppm, peaking in winter when temperatures were lowest. Chlorophyll-a exhibited wide variability (from < 1 to 25 µg L⁻¹), with maxima in late summer–autumn, surpassing the WHO low-risk recreational threshold (~10 µg L⁻¹). DOM values were moderate (optical density around 0.05–0.25 OD₄₅₀) but slightly higher in autumn, suggesting enhanced organic release from algal decay. Phycocyanin remained generally low (< 1 µg L⁻¹) yet increased during summer, consistent with cyanobacterial growth patterns.

In contrast, La Poza displayed overall lower variability and concentrations across all parameters: turbidity mostly < 10 NTU, temperature 7–24 °C, dissolved oxygen 9–13 ppm, and Chl-a rarely exceeding 10 µg L⁻¹. The more stable conditions suggest weaker stratification and lower phytoplankton productivity compared with Pucón. Overall, the figure highlights pronounced seasonal and spatial gradients, with Pucón showing stronger summer eutrophic signals and La Poza maintaining more oligotrophic characteristics.

Figure 3. Violin Diagram of water quality parameter in the sampling station Pucon and Poza.

## Evaluation of Models’ Performance

TimesFM and Chronos Large showed the strongest skill at the 1-day horizon and progressively degraded with lead time at both stations (Figures 4 and 5). At La Poza, D1 MAE was 0.099 µg/L for TimesFM and 0.106 µg/L for Chronos Large; D7 MAE increased to 0.300 and 0.275 µg/L, respectively. By D28, MAE increased to 0.702 µg/L for TimesFM and 0.757 µg/L for Chronos Large. At Pucón, D1 MAE was 0.063 µg/L for TimesFM and 0.071 µg/L for Chronos Large; D7 MAE was 0.241 and 0.275 µg/L, respectively; and D28 MAE reached 0.606 and 0.679 µg/L. These results indicate useful short-horizon performance but clear horizon-dependent degradation.

Figure 4. Forecast performance metrics for TimesFM and Chronos Large at La Poza. Metrics are computed by rolling-origin forecast horizon (1–30 days) from validated foundation-model predictions. Panels show MAPE, MSE, RMSE, bias, median absolute error, and MAE.

Figure 5. Forecast performance metrics for TimesFM and Chronos Large at Pucón. Metrics are computed by rolling-origin forecast horizon (1–30 days) from validated foundation-model predictions. Panels show MAPE, MSE, RMSE, bias, median absolute error, and MAE

3.3 Chl-a Time Series predictions

Forecast trajectories confirm that short-horizon predictions follow the broad Chl-a seasonal pattern, but they also show increasing amplitude damping and timing uncertainty with lead time (Figures 6 and 7). D7 lag diagnostics showed that the best correlation occurred at lag 0 for La Poza Chronos Large (r = 0.784), La Poza TimesFM (r = 0.774), and Pucón TimesFM (r = 0.860). Pucón Chronos Large was the exception, with the best D7 correlation at lag -14 days (best r = 0.790; lag-0 r = 0.743). Peak-date error was 14 days in all four D7 station/model combinations. These diagnostics indicate that D7 forecasts can capture the broad seasonal envelope but should not be interpreted as precise peak-timing forecasts.

Figure 6. TimesFM and Chronos Large Chl-a forecasts at La Poza for 1-, 7-, 14-, and 28-day horizons. The blue line is the accepted daily Chl-a target; red and green markers/lines are model predictions at rolling forecast origins

Figure 7. TimesFM and Chronos Large Chl-a forecasts at Pucón for 1-, 7-, 14-, and 28-day horizons. The blue line is the accepted daily Chl-a target; red and green markers/lines are model predictions at rolling forecast origins

3.4 Predictive uncertainty

We evaluated probabilistic forecast output using the cached q10 and q90 intervals from TimesFM and Chronos Large. At D7, empirical 10–90% interval coverage was 0.72 for La Poza TimesFM, 0.72 for La Poza Chronos Large, 0.88 for Pucón TimesFM, and 0.64 for Pucón Chronos Large, compared with the nominal 0.80 target. These results show that predictive intervals provide useful uncertainty information but are not uniformly calibrated across stations and models. Undercoverage for Pucón Chronos Large indicates that model-specific uncertainty should be interpreted cautiously for warning applications.

Figure 8. Predictive uncertainty intervals for D7 Chl-a forecasts. The blue line is the accepted daily Chl-a target, colored lines show q50 forecasts, and shaded envelopes show q10–q90 intervals for TimesFM and Chronos Large at Pucón and La Poza. Panel titles report empirical interval coverage and sample size.

3.5 Algal Bloom Behavior in the last thirty years

Table 1 shows the updated report of bloom behavior in the last thirty years in Villarrica Lake a based on information gathered from Vigilantes del Lago. Photographs of the blooms can be found in Supporting Information.

Table 1. Update report Blooms Villarrica Lake.

| Year | Reported blooms | Season | Algae Species | Group | Sector on the lake |
| --- | --- | --- | --- | --- | --- |
| 1993 | February | Summer | Microcystis aeruginosa | Cyanophyceae | South, South-West |
| 2005 | September 25 | Spring | - | Chlorophyceae | South Ribera |
| 2008 | 12 January, 6 November | Summer, Spring | Fragilaria sp | Bacillariophyceae | North Ribera |
| 2010 | 25, 30 January, 26 April | Summer, Autumn | Dolichospermum sp | Cyanophyceae | Coast Villarrica-Pucón |
| 2011 | 10 January | Summer | Dolichospermum sp | Cyanophyceae | La Poza and Pucón |
| 2012 | 24,25 February | Summer | Dolichospermum sp | Cyanophyceae | Pucón, La Poza |
| 2015 | 19 January | Summer | Dolichospermum sp | Cyanophyceae | Center |
| 2016 | 2 January | Summer | Dolichospermum sp | Cyanophyceae | South Ribera |
| 2017 | 10 February | Summer | Spirogyra sp | Charophyceae | South Ribera |
| 2018 | 25 January, 25 May | Summer, Autumn | Dolichospermum sp | Cyanophyceae | South |
| 2019 | 5 April | Autumn | Dolichospermum sp | Cyanophyceae | Pucón, La Poza |
| 2020 | 7, 20 January, 16 April | Summer, Autumn | Dolichospermum sp | Cyanophyceae | Pelagial Villarrica |
| 2021 | 3 january, 4,14 april, 21 may | Summer, autumn | Dolichospermum sp | Cyanophyceae | Pucon |
| 2022 | No reported |  |  |  |  |
| 2023 | 10 January, 2 may | Summer, Autumn | Dolichospermum sp | Cyanophyceae | Pucón, La Poza |

*Based on information from reports generated by Victor Duran of Vigilantes del Lago.

Figure 9 shows the seasonal variation of Chlorophyll-a (Chl-a, µg L⁻¹) concentration at two monitoring stations, Pucón and La Poza, throughout the year. Both panels illustrate different alert frameworks: the left graph corresponds to potabilization (drinking water) thresholds, and the right to recreational-use thresholds following the World Health Organization (WHO) guidance (WHO, 2021). For drinking water, the WHO recommends early-warning surveillance when Chl-a concentrations exceed 10 µg L⁻¹ (with dominance of cyanobacteria) and management actions when associated microcystin-LR levels approach the guideline value of 1 µg L⁻¹. For recreational waters, the WHO framework identifies a low-risk level near 10 µg L⁻¹ Chl-a (≈ 20 000 cells mL⁻¹) and a moderate-risk level around 100 µg L⁻¹ Chl-a (≈ 100 000 cells mL⁻¹). In both stations, Chl-a values remain low during most months, staying below 5 µg L⁻¹ from June to December. A distinct bloom peak occurs between March and April, when Pucón reaches concentrations above 20 µg L⁻¹—surpassing both the drinking-water and recreational low-risk thresholds—while La Poza shows lower yet noticeable increases around 9–10 µg L⁻¹ (WHO, 2021). These results indicate a pronounced seasonal pattern, with higher algal biomass in late summer to early autumn, suggesting that environmental conditions during this period (e.g., temperature, water column stratification, nutrient availability) favor phytoplankton growth.

Figure 9. Indicators of water alert levels for drinking and recreational.

Figure 10 presents quantitative indicators of cyanobacteria (Cyanophyceae cell density) at two monitoring stations, Pucón and La Poza, from 2021 to 2024, under two water-use categories: drinking water (left panel) and recreational water (right panel). According to the World Health Organization (WHO) guidelines, for drinking-water sources, surveillance should begin when cyanobacterial cell densities exceed 2 000 cells mL⁻¹ (≈ 2 × 10⁶ cells L⁻¹), while concentrations above 100 000 cells mL⁻¹ (≈ 1 × 10⁸ cells L⁻¹) indicate conditions unsuitable for potabilization without advanced treatment and toxin monitoring [49]. For recreational waters, the WHO framework defines a low-risk level near 20 000 cells mL⁻¹ (≈ 2 × 10⁷ cells L⁻¹) and a moderate-risk level around 100 000 cells mL⁻¹ (≈ 1 × 10⁸ cells L⁻¹), above which there is a significant probability of adverse health effects (Chorus and Welker, 2021).

In both cases, cyanobacterial abundance peaked sharply in 2022, reaching values near 1 × 10⁶ cells L⁻¹ (≈ 1 000 cells mL⁻¹) at both sites. These concentrations exceed the WHO surveillance threshold for drinking-water sources and approach the lower alert boundary for recreational exposure, as indicated by the shaded reference bands. After 2022, cyanobacteria levels dropped markedly by nearly two orders of magnitude—approaching negligible values by 2023–2024, thus returning well below the WHO surveillance limits. Similarly, in the recreational-use panel, 2022 also shows concentrations surpassing the alert level and entering the non-recreation range, reflecting potentially hazardous bloom conditions. From 2023 onward, cell counts declined substantially, indicating recovery toward safe recreational standards. Overall, the figure illustrates a transient cyanobacterial bloom event in 2022 at both stations, followed by a rapid decline in subsequent years, highlighting the need for continued monitoring and early-warning systems for water-use safety in Lake Villarrica.

Figure 10.  Quantitative indicators of water cyanobacteria for drinking and recreational.

- Discussion

The results of this study demonstrate the strong potential of combining satellite remote sensing, long-term in-situ monitoring, and advanced machine-learning techniques to improve the prediction and understanding of algal dynamics in Lake Villarrica. Over more than three decades of monitoring, the lake has exhibited recurrent episodes of cyanobacteria blooms—particularly dominated by Dolichospermum sp.—consistent with a system under increasing eutrophication pressure. The historical dataset revealed clear spatial differences between the Pucón and La Poza sites, with Pucón showing higher chlorophyll-a, greater turbidity, and more pronounced seasonal variability. These patterns align with previous findings that highlight stronger anthropogenic pressures and greater thermal stratification on the Pucón side of the lake, thus emphasizing the need for location-specific forecasting models (Rodríguez-López et al., 2025, 2024, 2023b).

Foundational forecasting models provided key insights into the predictability of Chlorophyll-a dynamics. The good performance at short horizons (1–7 days) of TimesFM and Chronos Large suggests that these architectures can effectively capture nonlinear interactions between environmental variables, even without task-specific training. This is a significant result, as few-shot or zero-shot frameworks minimize computational cost and eliminate dependence on frequent retraining, which is particularly advantageous for operational early-warning systems in freshwater environments. The degradation of model performance at horizons beyond 14 days reflects not a model deficiency but intrinsic limits of ecological predictability, echoing evidence that bloom dynamics respond to rapidly changing meteorological and hydrological drivers.

The lag analysis also supports a more cautious interpretation of the 7-day forecasts. For three of the four station/model combinations, maximum D7 correlation occurred at lag 0, suggesting no systematic temporal displacement in those cases. However, Pucón Chronos Large reached its best correlation at a non-zero lag, and all D7 cases showed non-negligible peak-date error. Therefore, the D7 forecasts should not be described as precisely resolving bloom timing. A more defensible interpretation is that they capture the direction and broad seasonal structure of Chl-a changes, while peak timing and peak amplitude remain uncertain. This distinction is important for operational warning systems, where a forecast that captures the general rise of Chl-a may still be useful, but only if managers understand that the exact timing of maximum concentration is uncertain.

The uncertainty analysis further shows that model reliability varies by station and model. The q10–q90 predictive intervals provide useful information, but empirical coverage is not uniformly calibrated. At the 7-day horizon, interval coverage was 0.72 for both TimesFM and Chronos Large at La Poza, 0.88 for TimesFM at Pucón, and 0.64 for Chronos Large at Pucón, compared with a nominal 0.80 interval. Thus, predictive uncertainty should be reported explicitly rather than implied from point forecasts alone. The undercoverage observed in some cases indicates that forecast intervals may be too narrow or that model uncertainty does not fully capture target reconstruction uncertainty, local bloom dynamics, or missing environmental drivers.

Several uncertainty sources must be distinguished. First, if satellite-derived pigment products are retained in the broader methodological framing, their retrieval uncertainty must be quantified separately through matchup validation, atmospheric correction quality control, cloud masking, and optical-water complexity assessment. Second, spatial mismatch can occur between point-based in-situ or buoy observations and satellite pixels, especially nearshore where gradients, mixed pixels, and shoreline effects may be important. Third, the daily reconstruction process itself introduces uncertainty because imputed values may smooth, shift, or attenuate short-lived blooms. Fourth, the forecasting models currently use univariate Chl-a context windows; they do not directly ingest meteorological, hydrodynamic, nutrient, or satellite-reflectance predictors. Consequently, medium-range forecast degradation should not be attributed only to “intrinsic ecological unpredictability.” It likely reflects a combination of ecological variability, missing exogenous drivers, reconstruction artifacts, and model horizon limitations.

The long-term in-situ data reveal clear spatial and seasonal differences in water quality in Lake Villarrica, which strongly influence the performance and interpretation of pigment retrieval and forecasting models. At Pucón, turbidity varied widely—from 1 NTU to more than 40 NTU, particularly in summer while temperature ranged from 6 °C in winter to 26–28 °C in summer, demonstrating strong thermal stratification. Chlorophyll-a at this site exhibited large variability, from <1 µg L⁻¹ to peaks exceeding 25 µg L⁻¹, surpassing the WHO low-risk recreational threshold of 10 µg L⁻¹. In contrast, La Poza displayed narrower ranges for turbidity (<10 NTU), DO (9–13 ppm), and Chlorophyll-a (typically <10 µg L⁻¹), indicating more oligotrophic and stable conditions. These quantified differences confirm that bloom-prone behavior in the lake is spatially heterogeneous, with Pucón consistently showing stronger eutrophic signals.

The historical bloom record further contextualizes these findings. Over the past 30 years, Lake Villarrica has experienced more than 15 documented bloom events dominated by cyanobacteria such as Dolichospermum sp. Peaks commonly occurred in January–April, consistent with observed Chlorophyll-a increases above 20 µg L⁻¹ at Pucón. The cyanobacteria cell-density analysis showed an exceptional spike in 2022, reaching ~1 × 10⁶ cells L⁻¹, exceeding the WHO drinking-water surveillance threshold (2 × 10⁶ cells L⁻¹ equivalent) and approaching the recreational low-risk boundary (~2 × 10⁷ cells L⁻¹ when expressed per WHO guidance). These extreme values contrast sharply with the near-zero cell densities recorded in 2023–2024, demonstrating the strong interannual variability typical of Patagonian lakes.

Overall, the integration of remote sensing, in-situ data, and advanced forecasting models provides a robust framework for understanding and predicting bloom dynamics in Lake Villarrica. The numerical results confirm that foundation models excel at short-term prediction but that medium-range forecasts are limited by system unpredictability rather than model architecture. Future progress will depend on coupling these models with hydrodynamic drivers, nutrient-load proxies, and meteorological forecasts to improve prediction of events exceeding 20–25 µg L⁻¹ of Chlorophyll-a or 10⁶ cells L⁻¹ of cyanobacteria. Considering ongoing climatic warming and increasing anthropogenic pressure in the basin, this integrated approach is essential for developing operational early-warning systems that protect ecosystem services, recreational safety, and water-resource management in Lake Villarrica.

- Conclusion

This study provides a reproducible evaluation of foundation time-series models for short-term Chl-a forecasting at two Lake Villarrica monitoring stations, a system that has exhibited recurrent cyanobacterial blooms—often dominated by Dolichospermum sp over more than three decades of monitoring. Clear spatial heterogeneity between Pucón and La Poza, with Pucón showing higher Chlorophyll-a levels, greater turbidity, stronger thermal stratification, and bloom peaks exceeding 25 µg L⁻¹, underscores the need for site-specific forecasting frameworks. TimesFM and Chronos Large showed strong short-horizon skill, with lowest errors at D1 and systematic degradation toward D14–D28 horizons. The revised analysis also shows that forecast interpretation must be reconstruction-aware: the daily target is not fully observed, and imputed/reconstructed values make up a substantial portion of the modeled daily series. Additional diagnostics demonstrate that D7 forecasts generally capture broad seasonal dynamics but do not guarantee exact peak timing; q10–q90 intervals are informative but not uniformly calibrated. Historical bloom records further contextualize these findings: more than 15 cyanobacterial bloom events have occurred in the lake since the 1990s, including an extreme spike in 2022 (~1 × 10⁶ cells L⁻¹), illustrating strong interannual variability typical of Patagonian lakes. Altogether, these results highlight that while foundation models provide robust short-term forecasting capabilities, improving medium-range prediction will require coupling machine-learning architectures with hydrodynamic processes, nutrient-load proxies, and meteorological forecasts. As anthropogenic pressures and climatic warming continue to intensify eutrophication risks, this integrated approach represents a critical foundation for developing operational early-warning systems that support ecosystem health, recreational safety, and sustainable water-resource management in Lake Villarrica.

# Conflict of Interest

The authors declare that the research was conducted in the absence of any commercial or financial relationships that could be construed as a potential conflict of interest.

# Author Contributions

Conceptualization, L.R-L.; methodology, L.R-L and D.B-U.; software, D.B-U, and L.B.A.; validation, L.R-L and D.B-U.; formal analysis, L.R-L.; investigation, L.R-L.; resources, F.F, L.B and R.U.; data curation, D.B-U and L.B-A.; writing—original draft preparation, L.R-L.; writing—review and editing, L.R-L, D.B-U, L.B, and. F.F; visualization, L.B.A supervision, R.U.; project administration, R.U and L.R-L.; funding acquisition, L.R-L and  R.U.

# Funding

Fondecyt Iniciación 2025 Project ANID/11250177.

# Acknowledgments

L.R.L thank to Fondecyt Iniciación 2025 Project ANID/11250177. L.R.-L. and R.U are grateful to the Centro de Recursos Hídricos para la Agricultura y la Minería (CRHIAM) (Project ANID/FONDAP/15130015 and ANID/FONDAP/1523A0001). Also we thanks to Vigilantes del Lago, especially Victor Duran.

References

Ananias, P.H.M., Negri, R.G., 2021. Anomalous behaviour detection using one-class support vector machine and remote sensing images: a case study of algal bloom occurrence in inland waters. Int. J. Digit. Earth 14, 921–942. https://doi.org/10.1080/17538947.2021.1907462

Ansari, A.F., Stella, L., Turkmen, C., Zhang, X., Mercado, P., Shen, H., Shchur, O., Rangapuram, S.S., Arango, S.P., Kapoor, S., Zschiegner, J., Maddix, D.C., Wang, H., Mahoney, M.W., Torkkola, K., Wilson, A.G., Bohlke-Schneider, M., Wang, Y., 2024. Chronos: Learning the Language of Time Series.

Aranda, A.C., Rivera-Ruiz, D., Rodríguez-López, L., Pedreros, P., Arumí-Ribera, J.L., Morales-Salinas, L., Fuentes-Jaque, G., Urrutia, R., 2021. Evidence of climate change based on lake surface temperature trends in south central Chile. Remote Sens. (Basel). 13. https://doi.org/10.3390/rs13224535

Arias, F., Zambrano, M., Galagarza, E., Broce, K., 2025. Mapping Harmful Algae Blooms: The Potential of Hyperspectral Imaging Technologies. Remote Sens. (Basel). https://doi.org/10.3390/rs17040608

Baek, S., Kim, W., 2025. Review on Hyperspectral Remote Sensing of Tidal Zones. Ocean Science Journal. https://doi.org/10.1007/s12601-024-00189-4

Beal, M.R.W., Özdoğan, M., Block, P.J., 2024. A Machine Learning and Remote Sensing-Based Model for Algae Pigment and Dissolved Oxygen Retrieval on a Small Inland Lake. Water Resour. Res. 60. https://doi.org/10.1029/2023WR035744

Bilous, O.P., Nezbrytska, I., Zhezherya, V., Dubniak, S., Batoh, S., Kazantsev, T., Polishchuk, O., Zhezherya, T., Leontieva, T., Cantonati, M., 2023. Interactions between Aquatic Plants and Cyanobacterial Blooms in Freshwater Reservoir Ecosystems. Water (Switzerland) 15. https://doi.org/10.3390/w15040672

Che, S., Du, G., Zhong, X., Mo, Z., Wang, Z., Mao, Y., 2023. Quantification of Photosynthetic Pigments in Neopyropia yezoensis Using Hyperspectral Imagery. Plant Phenomics 5. https://doi.org/10.34133/plantphenomics.0012

Chini Zittelli, G., Lauceri, R., Faraloni, C., Silva Benavides, A.M., Torzillo, G., 2023. Valuable pigments from microalgae: phycobiliproteins, primary carotenoids, and fucoxanthin. Photochemical and Photobiological Sciences 22, 1733–1789. https://doi.org/10.1007/s43630-023-00407-3

Chorus, I., Welker, M., 2021. Toxic Cyanobacteria in Water; A Guide to Their Public Health Consequences, Monitoring and Management; Second Edition.

Das, A., Kong, W., Sen, R., Zhou, Y., 2024. A decoder-only foundation model for time-series forecasting.

Dierssen, H.M., Ackleson, S.G., Joyce, K.E., Hestir, E.L., Castagna, A., Lavender, S., McManus, M.A., 2021. Living up to the Hype of Hyperspectral Aquatic Remote Sensing: Science, Resources and Outlook. Front. Environ. Sci. https://doi.org/10.3389/fenvs.2021.649528

Feng, L., Wang, Y., Hou, X., 2024. Harmful algal blooms in inland waters. Nat Rev Earth Environ 5, 631–644.

He, H.H., Li, X., Wang, D., Qiao, W., Sun, Y., Han, Y., Zhang, F., Zhao, X., 2025. A novel quad-modality deep neural network for estimating chlorophyll-a concentrations in Lianyungang’s lakes and reservoirs using Sentinel-2 MSI data. Water Res. 286. https://doi.org/10.1016/j.watres.2025.124246

Hyndman, R.J., Athanasopoulos, G., 2018. Forecasting: Principles and Practice.

Jaywant, S.A., Arif, K.M., 2024. Remote Sensing Techniques for Water Quality Monitoring: A Review. Sensors. https://doi.org/10.3390/s24248041

Khruschev, S.S., Plyusnina, T.Y., Antal, T.K., Pogosyan, S.I., Riznichenko, G.Y., Rubin, A.B., 2022. Machine learning methods for assessing photosynthetic activity: environmental monitoring applications. Biophys. Rev. https://doi.org/10.1007/s12551-022-00982-2

Li, M., Joehnk, K., Toscas, P., Garcia, L.R., Jin, H., Biswas, T.K., 2025. Forecasting Chlorophyll-a in the Murray–Darling Basin Using Remote Sensing. Remote Sens. (Basel). 17. https://doi.org/10.3390/rs17101684

Lim, B., Zohren, S., 2021. Time-series forecasting with deep learning: A survey. Philosophical Transactions of the Royal Society A: Mathematical, Physical and Engineering Sciences. https://doi.org/10.1098/rsta.2020.0209

Lima, T.M.A. de, Giardino, C., Bresciani, M., Barbosa, C.C.F., Fabbretto, A., Pellegrino, A., Begliomini, F.N., 2023. Assessment of Estimated Phycocyanin and Chlorophyll-a Concentration from PRISMA and OLCI in Brazilian Inland Waters: A Comparison between Semi-Analytical and Machine Learning Algorithms. Remote Sens. (Basel). 15. https://doi.org/10.3390/rs15051299

Ma, C., Dou, H., Zhao, Z., Qiu, X., Li, H., Wang, X., 2025. Review of in-situ non- and micro-destructive techniques for pigment analysis in architectural heritage. npj Heritage Science. https://doi.org/10.1038/s40494-025-01675-1

Mahmoud, E., Islam, A., El-Magd Editors, A., 2025. Springer Remote Sensing/Photogrammetry Modelling and Advanced Earth Observation Technologies for Coastal Zone Management.

Manzi, M.P., 2025. Optical remote sensing of sea water quality through a multi-sensor data-driven approach.

Molares, S., Morales, D., Aigo, J., Skewes, J.C., 2022. Cultural Limnology in Patagonia: Knowledge and Water Management in Mapuche Rural Communities, in: Freshwaters and Wetlands of Patagonia. Natural and Social Sciences of Patagonia. Springer, Cham, Switzerland.

Park, J., Patel, K., Lee, W.H., 2024. Recent advances in algal bloom detection and prediction technology using machine learning. Science of the Total Environment. https://doi.org/10.1016/j.scitotenv.2024.173546

Raffel, C., Shazeer, N., Roberts, A., Lee, K., Narang, S., Matena, M., Zhou, Y., Li, W., Liu, P.J., 2020. Exploring the Limits of Transfer Learning with a Unified Text-to-Text Transformer, Journal of Machine Learning Research.

Raghul, M., Porchelvan, P., 2025. An Approach for Monitoring Chlorophyll-A and Turbidity Levels in Saduperi and Kaveripakkam Lakes Located in and Around Vellore Region, Tamil Nadu, India, Using Spectral Unmixing Technique. Journal of the Indian Society of Remote Sensing 53, 711–730. https://doi.org/10.1007/s12524-024-02019-5

Rodríguez López, L., Bustos Usta, D.F., Duran-Llacer, I., Bravo Alvarez, L., Bourrel, L., Frappart, F., Urrutia, R., 2025. Advanced phycocyanin detection in a south American lake using landsat imagery and remote sensing. Frontiers in Remote Sensing 6, 1633522–1633538.

Rodríguez-López, L., Bustos Usta, D.F., Bravo Alvarez, L., Duran-Llacer, I., Bourrel, L., Frappart, F., Cardenas, R., Urrutia, R., 2024. Algal Pigment Estimation Models to Assess Bloom Toxicity in a South American Lake. Water (Switzerland) 16. https://doi.org/10.3390/w16243708

Rodríguez-López, L., Duran-Llacer, I., Bravo Alvarez, L., Lami, A., Urrutia, R., 2023a. Recovery of Water Quality and Detection of Algal Blooms in Lake Villarrica through Landsat Satellite Images and Monitoring Data. Remote Sens. (Basel). 15. https://doi.org/10.3390/rs15071929

Rodríguez-López, L., Duran-Llacer, I., Bravo Alvarez, L., Lami, A., Urrutia, R., 2023b. Recovery of Water Quality and Detection of Algal Blooms in Lake Villarrica through Landsat Satellite Images and Monitoring Data. Remote Sens. (Basel). 15. https://doi.org/10.3390/rs15071929

Rodríguez-López, L., González-Rodríguez, L., Duran-Llacer, I., García, W., Cardenas, R., Urrutia, R., 2022. Assessment of the Diffuse Attenuation Coefficient of Photosynthetically Active Radiation in a Chilean Lake. Remote Sens. (Basel). 14. https://doi.org/10.3390/rs14184568

Rodríguez-López, L., Usta, D.F.B., Duran-Llacer, I., Alvarez, L.B., Bourrel, L., Frappart, F., Urrutia, R., 2025. Advanced phycocyanin detection in a south American lake using landsat imagery and remote sensing. Frontiers in Remote Sensing 6. https://doi.org/10.3389/frsen.2025.1633522

Rolim, S.B.A., Veettil, B.K., Vieiro, A.P., Kessler, A.B., Gonzatti, C., 2023. Remote sensing for mapping algal blooms in freshwater lakes: a review. Environmental Science and Pollution Research. https://doi.org/10.1007/s11356-023-25230-2

Savitzky, A., Golay, M.J., 1964. Smoothing and differentiation of data by simplified least squares procedures. Anal. Chem. 8, 1627–1639.

Singh, P.K., Kumar, U., Kumar, I., Dwivedi, A., Singh, P., Mishra, S., Seth, C.S., Sharma, R.K., 2024. Critical review on toxic contaminants in surface water ecosystem: sources, monitoring, and its impact on human health. Environmental Science and Pollution Research 31, 56428–56462. https://doi.org/10.1007/s11356-024-34932-0

Sonmez, M.E., Eczacıoglu, N., Gumuş, N.E., Aslan, M.F., Sabanci, K., Aşikkutlu, B., 2022. Convolutional neural network - Support vector machine based approach for classification of cyanobacteria and chlorophyta microalgae groups. Algal Res. 61. https://doi.org/10.1016/j.algal.2021.102568

Srivastava, M.K., Gaur, S., Ohri, A., Srivastava, P.K., Singh, N., 2023. Applications of remote sensing in water quality assessment, in: Remote Sensing in Precision Agriculture: Transforming Scientific Advancement into Innovation. Elsevier, pp. 217–236. https://doi.org/10.1016/B978-0-323-91068-2.00019-9

Wang, H., Convertino, M., 2023. Algal bloom ties: Systemic biogeochemical stress and Chlorophyll-a shift forecasting. Ecol. Indic. 154. https://doi.org/10.1016/j.ecolind.2023.110760

WHO, 2021. World Health Organization. global water, sanitation and hygiene: annual report 2020.

Wu, J., Cao, Y., Wu, S., Parajuli, S., Zhao, K., Lee, J., 2025. Current Capabilities and Challenges of Remote Sensing in Monitoring Freshwater Cyanobacterial Blooms: A Scoping Review. Remote Sens. (Basel). https://doi.org/10.3390/rs17050918

Xiong, J., Lin, C., Cao, Z., Hu, M., Xue, K., Chen, X., Ma, R., 2022. Development of remote sensing algorithm for total phosphorus concentration in eutrophic lakes: Conventional or machine learning? Water Res. 215. https://doi.org/10.1016/j.watres.2022.118213

Yang, X., Huang, P.I., 2023. Improvements in the relationship between tropical precipitation and sea surface temperature from CMIP5 to CMIP6. Clim. Dyn. 60, 3319–3337.